# Gemini FPGA Programming Procedure

# Gemini_FPGAProg_ATP-828-62

Purpose: 	This procedure is used for programming the Hydra FPGA on the Gemini.

Scope:	This procedure is used during troubleshooting, or as an alternative bring-up step if pre-programmed FPGAs are unavailable.

Audience:	Test engineering and technicians.

Training:	None.

Revision History

(Most Recent First)

| Date | Author | Description |
| --- | --- | --- |
| 15-Aug-2018 | Jeff Bouchard | - Initial release |

# Requirements

- Altera USB blaster, or Terasic USB blaster

- Computer w/Quartus installed (see appendix A) and USB blaster driver installed (see appendix B)

- 359-591-13 connector, or equivalent 10-pos RoHS header

- 090-489-00 cable, or equivalent IEC power cable

- Gemini Hydra FPGA programming file (.pof) obtained through team center

# Hardware Setup

- Solder the 359-591-13 connector to J18 on the Gemini PCBA using lead-free solder. Note, you will need to remove the PCBA from the HLA to solder the header, then reassemble the HLA with the PCBA.

- Connect the USB blaster to the computer, via USB, and to the Gemini, via J18. Note the orientation of pin 1 on the cable (red wire on Terasic USB blaster, white silk-screen on Altera USB blaster). Match this to pin 1 on J18.

- Connect the IEC power cable to the Gemini HLA power cable. Then connect the other end to a power source to turn on the Gemini HLA.

# Programming Procedure

- Launch Quartus Prime Programmer (Start  type “Quartus Prime 16.1 Programmer”  select Quartus Prime 16.1 Programmer).

- Select Hardware Setup

- Select USB-Blaster [USB-0] from the currently selected hardware drop-down menu. Then select USB-Blaster in the available hardware items field. Then select close.

- Select Add File.

- Select the .pof programming file, then Open.

- Select all three checkboxes under Program/Configure. Then select Start.

- A message will display in the messages log stating “programming device” and the progress bar will show the percentage complete.

- When completed, a message will display in the messages log stating “ended programmer operation” and the progress bar will state 100% (Successful).

- Close Quartus.

# Equipment Clean Up

- Unplug the IEC power cable from the power source to turn off the Gemini HLA.

- Disconnect the USB blaster from J18 on the Gemini and from the computer USB.

- Remove the 359-591-13 header from J18. Note, the Gemini PCBA will need to be removed from the HLA in order to de-solder the connector, then the HLA will need to be reassembled with the PCBA.

# Appendix A: Quartus Installation

- Run the Quartus installer as administrator from the location specified below. Follow the on-screen prompts for installation, selecting defaults and accepting any license agreements.

Teradyne:	\\ter.teradyne.com\projects_ah\npiw\Dragon\development\Pilot Line Software\Release\Quartus

Flex: 		\\agate\NPI Projects\UltraFlex2\SW\Uflex2IGXLsync\Quartus

# Appendix B: USB Blaster Driver Installation

## Windows 10

- Plug the USB-Blaster into your PC.

- Open the Device and Printers (Control Panel | Devices and Printers).

- Under Unspecified, USB Blaster should be listed. Right mouse click on this and then select Properties.

- Select the Hardware tab and select Properties.

- A new window should pop up with the General tab already selected. Select Change Settings.

- Again a new window should pop up with the General tab already selected. Select Update.

- Select browse my computer for driver software.

- Find <Path to Quartus II installation>\quartus\drivers\

- (Note 1: Your altera file is located at the location you selected when you first installed Quartus. The location listed in this document is the default location)

- (Note 2: Stop at the drivers folder, i.e., do NOT go deeper by opening a folder within the drivers folder)

- Select OK. Make sure the proper path was selected then select next.

- If the Windows security window pops up Check the Always trust software from “Altera Corporation” box and select Install.

## Windows 7

- Plug the USB-Blaster download cable into the PC. The following will appear on the screen: Device driver software was not successfully install. Select Click here for details.

- Select Change settings…

- Now cancel this by selecting Cancel

- Open the Device Manager (Windows Key | Control Panel | Device Manage)

- Under Other devices, USB Blaster should be listed. Right mouse click on this and then select Update Driver Software…

- Select browse my computer for driver software

- Find <Path to Quartus II installation>\quartus\drivers\, then select Next; (Note: Stop at this folder, i.e., do NOT go deeper by opening a folder within the drivers folder)

- Check the Always trust … box and then select Install